Week #1

Study and understand the basic networking tools - Wireshark, Tcpdump, Ping, Traceroute.

Learn and Understand Network Tools - Wireshark - Perform and analyze Ping PDU capture - Examine HTTP packet capture - Analyze HTTP packet capture using filter - Tcpdump - Capture packets - Ping - Test the connectivity between 2 systems - Traceroute - Perform traceroute checks - Nmap - Explore an entire network

IMPORTANT INSTRUCTIONS:

- This manual is written for Ubuntu Linux OS only. You can also execute these experiments on VirtualBox or VMWare platform.

- For few tasks, you may need to create 2 VMs for experimental setup.

- Perform sudo apt-get update before installing any tool or utility.

- Install any tool or utility using the command sudo apt-get install name_of_the_tool

- Take screenshots wherever necessary and upload it to Edmodo as a single PDF file.

(Refer general guidelines for submission requirements).

- To define an IP address for your machine (e.g., Section – ‘a’ & Serial number is 1, then your IP address should be 10.0.1.1. Section – ‘h’ & & Serial number is 23, then your IP address should be 10.0.8.23) – applicable only for relevant tasks (which doesn’t requires internet connectivity to execute the tasks).

Task 1: Linux Interface Configuration (ifconfig / IP command) Step 1: To display status of all active network interfaces.

ifconfig (or) ip addr show Analyze and fill the following table: ip address table:

| Interface name | IP address (IPv4 / IPv6) | MAC address |  |
| --- | --- | --- | --- |
| lo | IPV4: 127.0.0.1/8 | 00:00:00:00:00:00 |  |
| enp0s31f6 | IPV4:10.0.20.46 | 90:8d:6e:8c:65:e3 |  |

Step 2: To assign an IP address to an interface, use the following command. sudo ifconfig interface_name 10.0.your_section.your_sno netmask 255.255.255.0 (or) sudo ip addr add 10.0.your_section.your_sno /24  dev interface_name

Step 3: To activate / deactivate a network interface, type.  	sudo ifconfig interface_name down  	sudo ifconfig interface_name up

Step 4: To show the current neighbor table in kernel, type

ip neigh

# Task 2: Ping PDU (Packet Data Units or Packets) Capture

Step 1: Assign an IP address to the system (Host).

Note: IP address of your system should be 10.0.your_section.your_sno.

Step 2: Launch Wireshark and select ‘any’ interface

Step 3: In terminal, type ping 10.0.your_section.your_sno

## Observations to be made

Step 4: Analyze the following in Terminal

- TTL

- Protocol used by ping

- Time

Step 5: Analyze the following in Wireshark

On Packet List Pane, select the first echo packet on the list. On Packet Details Pane, click on each of the four “+” to expand the information. Analyze the frames with the first echo request and echo reply and complete the table below.

| Details | First Echo Request | First Echo Reply |
| --- | --- | --- |
| Frame Number | 1 | 2 |
| Source IP address | 10.0.4.17 | 10.0.4.17 |
| Destination IP address | 10.0.4.17 | 10.0.4.17 |
| ICMP Type Value | 8 | 0 |
| ICMP Code Value | 0 | 0 |
| Source Ethernet Address | 00:00:00:00:00:00 | 00:00:00:00:00:00 |
| Destination Ethernet Address | 00:00:00:00:00:00 | 00:00:00:00:00:00 |
| Internet Protocol Version | 4 | 4 |
| Time To Live (TTL) Value | 64 | 64 |

# Task 3: HTTP PDU Capture

Using Wireshark’s Filter feature

Step 1: Launch Wireshark and select ‘any’ interface. On the Filter toolbar, type-in ‘http’ and press enter

Step 2: Open Firefox browser, and browse www.flipkart.com

## Observations to be made

Step 3: Analyze the first (interaction of host to the web server) and second frame (response of server to the client). By analyzing the filtered frames, complete the table below:

| Details | First Echo Request | First Echo Reply |
| --- | --- | --- |
| Frame Number | 31 | 33 |
| Source Port | 10.0.2.15 | 23.46.187.17 |
| Destination Port | 23.46.187.17 | 10.0.2.15 |
| Source IP address | IPv4 | IPv4 |
| Destination IP address | 0x800 | 0x800 |
| Source Ethernet Address | PcsCompu_73:c4:03 (08:00:27:c4:03) | RealtekU_12:35:02 (52:54:00:12:35:02) |
| Destination Ethernet Address | RealtekU_12:35:02 (52:54:00:12:35:02) | PcsCompu_73:c4:03 (08:00:27:c4:03) |

Step 4: Analyze the HTTP request and response and complete the table below.

| HTTP Request |  | HTTP Response |  |
| --- | --- | --- | --- |
| Get | GET / HTTP / 1.1 | Server | Apache (ubuntu |
| Host | connectivity-check.ubuntu.com | Content-Type | text/plain |
| User-Agent | Mozilla (Ubuntu) | Date | 16/1/23 |
| Accept-Language | /* | Location |  |
| Accept-Encoding | * | Content-Length | 8\r\n |
| Connection | close | Connection | keep-alive\r\n |

Using Wireshark’s Follow TCP Stream

Step 1: Make sure the filter is blank. Right-click any packet inside the Packet List Pane, then select ‘Follow TCP Stream’. For demo purpose, a packet containing the HTTP GET request

“GET / HTTP / 1.1” can be selected.

Step 2: Upon following a TCP stream, screenshot the whole window.

# Task 4: Capturing packets with tcpdump

Step 1: Use the command tcpdump -D to see which interfaces are available for capture.

sudo tcpdump -D

Step 2: Capture all packets in any interface by running this command:

## sudo tcpdump -i any

Note: Perform some pinging operation while giving above command. Also type www.google.com in browser.

## Observation

Step 3: Understand the output format.

Step 4: To filter packets based on protocol, specifying the protocol in the command line. For example, capture ICMP packets only by using this command:

## sudo tcpdump -i any -c5 icmp

Step 5: Check the packet content. For example, inspect the HTTP content of a web request like this:

## sudo tcpdump -i any -c10 -nn -A port 80

Step 6: To save packets to a file instead of displaying them on screen, use the option -w:  	 	sudo tcpdump -i any -c10 -nn -w webserver.pcap port 80

# Task 5: Perform Traceroute checks

Step 1: Run the traceroute using the following command.

## sudo traceroute www.google.com

Step 2: Analyze destination address of google.com and no. of hops

Step 3: To speed up the process, you can disable the mapping of IP addresses with hostnames by using the -n option

## sudo traceroute -n www.google.com

Step 4: The -I option is necessary so that the traceroute uses ICMP.  	sudo traceroute -I www.google.com

Step 5: By default, traceroute uses icmp (ping) packets. If you’d rather test a TCP connection to gather data more relevant to web server, you can use the -T flag.

sudo traceroute -T www.google.com

# Task 6: Explore an entire network for information (Nmap)

Step 1: You can scan a host using its host name or IP address, for instance.

nmap www.pes.edu

Step 2: Alternatively, use an IP address to scan.

## nmap 163.53.78.128 Step 3: Scan multiple IP address or subnet (IPv4)  	nmap 192.168.1.1 192.168.1.2 192.168.1.3

Questions on above observations:

- Is your browser running HTTP version 1.0 or 1.1? What version of HTTP is the server?

Both the server and browser are running on 1.1

- When was the HTML file that you are retrieving last modified at the server?

- How to tell ping to exit after a specified number of ECHO_REQUEST packets?

Use Ctrl+C to exit

- How will you identify remote host apps and OS?

Use ip neigh

TASK 7 A): NETCAT AS CHAT TOOL

a) Intra system communication (Using 2 terminals in the same system)

Step 1: Open a terminal (Ctrl+Alt+T). This will act as a Server.

Step 2: Type nc -l any_portnum (For eg., nc -l 1234)

Note: It will goto listening mode

Step 3: Open another terminal and this will act as a client.

Step 4: Type nc <your-system-ip-address> portnum

Note: portnum should be common in both the terminals (for eg., nc 10.0.2.8 1234)

Step 5: Type anything in client will appear in server

TASK 7 B): USE NETCAT TO TRANSFER FILES

The netcat utility can also be used to transfer files.

Step 1: At the server side, create an empty file named ‘test.txt’

sudo nc -l 555 > test.txt

Step 2: At the client side, we have a file ‘testfile.txt’. Add some contents to it.

Step 3: Run the client as:

sudo nc 10.0.2.8 555 < testfile.txt

here, 10.0.2.8 is the IP address of server and 555 is the port number.

Step 4: At server side, verify the file transfer using the command

cat test.txt

TASK 7 C): OTHER COMMANDS

1) To test if a particular TCP port of a remote host is open.

nc -vn 10.0.2.8 555

2) Run a web server with a static web page.

Step 1: Run the command below on local host (e.g. 10.0.2.8) to start a web server that

serves test.html on port 80.

while true; do sudo nc -lp 80 < test.html; done

Step 2: Now open http://10.0.2.8/test.html from another host to access it.

Step 3: Observe the details on the terminal

Name : Atharva Menkudle

SRN: PES2UG21CS104

Sec: B